# National University of Computer and Emerging Sciences

Lab Manual # 03

Digital Logic Design (EL1005)

| Course Instructor | Mr. Haris Jamil |
| --- | --- |
| Lab Instructor(s) | Mr. ZummarSaad Mr. Muhammad Salman Mubarik |
| Section | BDS-2A |
| Semester | Spring 2022 |

Department of Computer Science

FAST-NU, Lahore, Pakistan

In Lab Statement:

Operation of AND gate according to its truth table

Install IC (integrated circuit) for AND (74LS08) on the breadboard according to the connection diagram given below.

Operation of OR gate according to its truth table

Install IC (integrated circuit) for OR (74LS32) on the breadboard according to the connection diagram given below.

Operation of NOT gate according to its truth table

Install IC (integrated circuit) for NOT (74LS04) on the breadboard according to the connection diagram given below.

Question # 1:

Find Max terms from the following Min terms

F(X,Y,Z) =Σm(0,1,3,6,7)

- Write the expression for POS

- The numeric values as: Π M = ________

- Implement the max terms expression (part a) on Trainer and logic works.

Question # 2:

From the following truth table, “F” represents the output.

- Write the sum of min terms expression.

- Write product of max terms expression.

- Implement part (a) on Trainer and Logic Works.

Question # 3:

Convert the following expressions into SPOS and implement the final expression on Trainer and Logic Works.

(A’+B+C’) . (A+C’)

Question # 4:

For the Boolean function  F1(A,B,C)=m(0,2,5,6,7)  do the following:

- Find truth table

- Find minimal SOP expression for Boolean function F1.

- Fill the following table and implement the circuit

| IC type | Required No. of Gates | Gates per IC | Required No. of ICs |
| --- | --- | --- | --- |
| Total no. of ICs |  |  |  |