# National University of Computer and Emerging Sciences

Lab Manual # 01

Digital Logic Design (EL1005)

| Course Instructor | Mr. Haris Jamil |
| --- | --- |
| Lab Instructor(s) | Mr. Zummar Saad Mr. Salman Mubarik |
| Section | BDS-2D |
| Semester | Spring 2022 |

Department of Computer Science

FAST-NU, Lahore, Pakistan

Objectives:

- To learn and understand the working of NAND gate and NOR gate

- SOP expressions from K-map

Introduction to NAND Gate

74LS00 IC contains four 2-input NAND gates. The function table and connection diagram for this IC are shown below:

Function TableConnection Diagram:

| Inputs |  | Output |
| --- | --- | --- |
| A | B | Y |
| L | L | H |
| L | H | H |
| H | L | H |
| H | H | L |

H= Logic High, L= Logic Low

Introduction to NOR Gate

74LS02 IC contains four 2-input NOR gates. The function table and connection diagram for this IC are shown below:

Function Table:Connection Diagram:

| Inputs |  | Output |
| --- | --- | --- |
| A | B | Y |
| L | L | H |
| L | H | L |
| H | L | L |
| H | H | L |

H= Logic High, L= Logic Low

NOR and NAND Implementation on logic trainer:

Question#1: Implement the ANDand OR gate using only the NAND gates

- Z=A.B                   b. X=A+B			 c. XOR Gate

Question#2: Implement the AND and OR gate using only the NOR gates

- Z=A.B                   b. X=A+B			c. XNOR Gate

Question # 3:For the Boolean function  F1(A,B,C,D)=m(2,4,12,14)  do the following:

- Find truth table

- Find minimal SOP expression for Boolean function F1 using K-map. Draw K-map in the space given below. Implement it on logic trainer.