Lab 9

## Objectives:

To learn and understand the working of Half Adder, Full Adder and Latches

Adder and subtractor

Half Adder:

Half adder is a logic circuit that performs binary addition of two 1-bit numbers. It generates two outputs namely ‘Sum’ and ‘Carry’.

Truth Table:

| A | B | Carry | Sum |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

Boolean Expressions of Outputs:

Sum=A⊕B

Carry=AB

Half Subtractor:

Half subtractor is a logic circuit that performs binary subtraction of two 1-bit numbers. It generates two outputs namely ‘Difference’ and ‘Borrow’.

Truth Table:

| A | B | Borrow | Difference |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 |

Boolean Expressions of Outputs:

Difference=A⊕B

Borrow= AB

Full Adder:

Full adder is a logic circuit that performs binary addition of two 2-bit numbers. It generates two outputs namely ‘Sum’ and ‘Carry’.

Truth Table:

| A | B | C | Carry | Sum |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Boolean Expressions of Outputs:

Sum=A⊕B⊕C

Carry= AB+BC+AC  orCarry= AB+C(A⊕B)

Full Subtractor:

Full subtractor is a logic circuit that performs binary subtraction of two 2-bit numbers. It generates two outputs namely ‘Difference’ and ‘Borrow’.

Truth Table:

| A | B | C | Borrow | Difference |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Boolean Expressions of Outputs:

Difference=A⊕B⊕C

Borrow= AB+BC+AC  orCarry= AB+C(A⊕B)

Lab Tasks:

Question 1

Implement an SR latch on Logic Trainer and Logic Works with control input as follows:

Question 2:

Implement 2 bit by 2 bit multiplier cirucit on the Logic Trainer and Logic Works.

Question 3

Implement the full adder circuit on Logic Trainer and Logic Works.