Lab 6

OBJECTIVES:

- To learn and understand the working of different types of decoders

- To learn and understand how to design a multiple output combinational circuit using Decoders

EQUIPMENT:  Logic trainer, Logic probe

COMPONENTS: ICs 74LS08, 74LS32, 74LS04, 74LS139

2-to-4 line decoders:

74LS139 IC contains two fully independent 2-to-4 line decoders with active low enables. The function table and connection diagram for this IC are shown below:

Function Table:

| Enable | Selection Inputs |  | Outputs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| G | B | A | Y0 | Y1 | Y2 | Y3 |
| H | X | X | H | H | H | H |
| L | L | L | L | H | H | H |
| L | L | H | H | L | H | H |
| L | H | L | H | H | L | H |
| L | H | H | H | H | H | L |

H= Logic High, L= Logic Low, X= Don’t Care

Connection Diagram:

3-to-8 line decoders:

74LS138 IC contains 3-to-8 line decoder. The function table and connection diagram for this IC are shown below:

Connection Diagram:

Lab Tasks:

Question 1 (Hardware Implementation):

Make a truth table and implement on Logic Works first 2 to 4 Decoder with a low enable using 8 AND gates and 3 NOT gates.

Question 2 (Hardware Implementation):

Implement on Logic Works first 3 to 8 decoder using two 2 to 4 decoders and a NOT gate

Question 3:

Implement on Logic Works a doubler circuit. It takes a 3 bit input and multiplies it by two and gives a 4 bit output using decoders and other logic gates.