# Homework

Following are the design problems. You must identify and mention the inputs and according to the logic what would be the output.

Represent the following logical problems in Boolean expression. Also, draw the circuit diagram.

Question # 1:

Develop a logic circuit with four input variables that will only produce a 1 output when exactly three input variables are 1s.

Question # 2:

Amna, Bilal and Rahim all are colleagues at work. Their work is to be divided in such a way that either only Amna works, or Bilal and Rahim work together and Amna does not work, or Amna and Bilal work together. Model this problem as a Boolean function and implement by combinational logic circuit.

Question # 3:

A burglar alarm is to sound if a master switch is on and either a light beam is broken (logic-0) or a pressure pad is stood on (logic-1). Model this problem as a Boolean function and implement by combinational logic circuit.

Question # 4:

An aircraft door is locked by two bolts A & B, each of which operates a sensor giving logic 1 when the bolt is fully inserted. A further sensor C gives logic 0 output when the door is shut. A circuit using logic gates is required to give logic output 1 only when both bolts are inserted, and the door is shut. Draw a logic circuit system showing how this aircraft door can be operated this way.