| Information Security (BDS-7A) – (CLO - 1) Quiz 05 (Fall 2024). Instructor: Dr. Syed M. Irteza |  | Name: |
| --- | --- | --- |
| Date: 2024-11-29 |  | Roll Number: |
| Total Marks: 10 | Time Allowed: 6 mins |  |

- Which of the following protocols is responsible for providing authentication and integrity in IPsec? (2m)

  - AH (authentication header)

  - ESP (encapsulating security payload)

  - IKE (Internet Key Exchange)

  - Both AH and ESP ***

- What is the primary purpose of a Security Parameter Index (SPI) in an IPsec Security Association (SA)? (2m)

  - To encrypt the IP header

  - To authenticate the source of a packet

  - To uniquely identify a specific SA ***

  - To determine the lifetime of an SA

- Tunnel mode would be used in IPsec if we want a secure _________ between _______. (2m)

  - virtual channel; gateway and gateway ***

  - virtual channel; endhost and endhost

  - real channel; endhost and endhost

  - real channel; gateway and endhost

- The SSL Record Protocol takes application data and does the following (in correct order): (2m)

  - padding, encryption, compression, MAC computation, fragmentation, and adds a header

  - compression, MAC computation, padding, encryption, adds a header, and fragmentation

  - fragmentation, MAC computation, encryption, compression, padding, and adds a header

  - fragmentation, compression, MAC computation, padding, encryption, and adds a header ***

- What are the main purposes of an SSL Handshake? (2m)

Agree on a set of algorithms for confidentiality, integrity and authentication (1 mark)

1 mark for mentioning any one of the following:

- Exchange random numbers between the client and the server to be used for the subsequent generation of the keys

- Establish a symmetric key by means of public key operations, e.g., RSA

- Negotiate the session-ID

- Exchange the necessary certificates