Q1. Answer the following?

a)	Ret 3 is:

1.	Valid statement

2.	Invalid statement

b)	Suppose SP=0xFFFE, if the above statement is valid what will be its new value? If its not valid write why?

_0x0003__

Q3. Write a subroutine that capitalizes each starting character of a sentence on video screen. For sake of simplicity lets define that a sentence only starts after a full stop followed by space. Ignore other conditions. (ascii of dot is 0x2E and space is 0x20) [5 Marks]

[org 0x0100]

jmp start

capital:

push bp

mov bp, sp

push di

push 0xb800

pop es

mov di, 0

l1:

cmp byte[es:di], 0x2E

jne skip

cmp byte[es:di+2], 0x20

jne skip

cmp byte[es:di+4], 0x61

jl skip

cmp byte[es:di+4], 0x7A

jg skip

sub byte[es:di+4], 0x20

skip:

add di, 2

cmp di, 3996

jl l1

pop di

pop bp

ret

start:

;;;;code to test the subroutine;;;;;;

push 0xb800

pop es

mov di, 120

mov cx, 10

mov ah, 0x07

mov si, 0

l2:

mov al, [strr+si]

mov [es:di], ax

add si, 1

add di, 2

loop l2

;;;;subroutine

call capital

mov ax, 0x4c00

int 21h

strr: db '. abc. efg'

Q3. Write a subroutine that swaps two rows of video screen. The subroutine takes two parameters: i.e. row numbers. Write complete subroutine. [5 Marks]

[org 0x0100]

jmp start

swap:

push bp

mov bp, sp

push ax

push si

push di

push cx

push dx

mov ah, 0

mov al, 80

mul byte[bp+4]

shl ax, 1

mov si, ax

mov ah, 0

mov al, 80

mul byte[bp+6]

shl ax, 1

mov di, ax

push 0xb800

pop es

mov cx, 80

l1:

mov ax, [es:si]

mov dx, [es:di]

mov [es:si], dx

mov [es:di], ax

add si, 2

add di, 2

loop l1

pop dx

pop cx

pop di

pop si

pop ax

pop bp

ret 4

start:

push 3

push 10

call swap

mov ax, 0x4c00

int 21h