Introduction to DOSBox Lab 10

Computer Organization and Assembly Language

| Student Name | Muhammad Hassan Khalid |
| --- | --- |
| Registration # | 21L-5692 |
| Instructor | Hazoor Ahmad |
| Class | CS3 |
| Section | A1, A2, D1, D2 |
| Semester | Fall 2022 |

Fast School of Computing

FAST-NU, Lahore, Pakistan

# Activity 1

## Assembly Language Code

[org 0x0100]

jmp start

message1: db 'Hi! I am Muhammad Hassan Khalid',0

message2: db 'I am Sad',0

message3: db 'Because I study at FAST',0

message4: db 'My Roll Number is 21L-5692',0

clrscr:

push es

push ax

push cx

push di

mov ax, 0xb800

mov es, ax

xor di, di

mov ax, 0x0720

mov cx, 2000

cld

rep stosw

pop di

pop cx

pop ax

pop es

ret

strlen:

push bp

mov bp, sp

push es

push cx

push di

les di, [bp+4]

mov cx, 0xffff

xor al, al

repne scasb

mov ax, 0xffff

sub ax, cx

dec ax

pop di

pop cx

pop es

pop bp

ret 4

printstr:

push bp

mov bp, sp

push es

push ax

push cx

push si

push di

push ds

mov ax, [bp+4]

push ax

call strlen

cmp ax, 0

jz exit

mov cx, ax

mov ax, 0xb800

mov es, ax

mov al, 80

mul byte [bp+8]

add ax, [bp+10]

shl ax, 1

mov di, ax

mov si, [bp+4]

mov ah, [bp+6]

cld

nextchar:

lodsb

stosw

loop nextchar

exit:

pop di

pop si

pop cx

pop ax

pop es

pop bp

ret 8

start:

call clrscr

mov ah, 0

INT 0x16

mov ax, 30

push ax

mov ax, 10

push ax

mov ax, 0x71

push ax

mov ax, message1

push ax

call printstr

mov ah, 0

INT 0x16

mov ax, 30

push ax

mov ax, 11

push ax

mov ax, 0x71

push ax

mov ax, message2

push ax

call printstr

mov ah, 0

INT 0x16

mov ax, 30

push ax

mov ax, 12

push ax

mov ax, 0x71

push ax

mov ax, message3

push ax

call printstr

mov ah, 0

INT 0x16

mov ax, 30

push ax

mov ax, 13

push ax

mov ax, 0x71

push ax

mov ax, message4

push ax

call printstr

mov ax, 0x4c00

int 0x21

## Debugging Screenshots

# Activity 2

## Assembly Language Code

[org 0x0100]

jmp code

w equ 70 ; width offset

x equ 50 ; starting x coordinate of line

y equ 100 ; starting y coordinate of line

c equ 60 ; color

code: mov ah, 0

mov al, 13h

int 10h

; draw diagonal 11:

mov cx, x

mov dx, y

mov al, c

u1: inc dx

mov ah, 0ch ; put pixel

int 10h

inc cx

cmp cx, x+w

jbe u1

;wait for keypress

mov ah,00

int 16h

; draw diagonal 12:

mov cx, x

mov dx, y

mov al, c

u11: dec dx

mov ah, 0ch ; put pixel

int 10h

inc cx

cmp cx, x+w

jbe u11

;wait for keypress

mov ah,00

int 16h

; draw diagonal 13:

mov cx, x+w

mov dx, y-w

mov al, c

u13: inc cx

mov ah, 0ch ; put pixel

int 10h

inc dx

cmp dx, y

jbe u13

;wait for keypress

mov ah,00

int 16h

; draw diagonal 14:

mov cx, x+w+w

mov dx, y

mov al, c

u12: dec cx

mov ah, 0ch ; put pixel

int 10h

inc dx

cmp dx, y+w

jbe u12

;wait for keypress

mov ah,00

int 16h

mov ah,00

int 16h

mov ax, 0x4c00

int 21h

## Debugging Screenshots